XY-050P-D User Manual

Product Overview:

4 working modes: DMX mode, Control mode, Auto mode, Slave mode

Channel mode: 21/24 channels

Pan: 540 degrees

Tilt: 270 degrees

LED dimming: Smooth dimming from 0-100%

Functions: Dimming, strobe, color wheel, CMY, fixed gobo wheel, rotating gobo wheel, gobo rotation, Iris, prism (with self-rotation), linear prism (with self-rotation), focus, zoom

Protocol: DMX512/RDM/ArtNet/sACN/

- Menu

| Menu 1 | Menu 2 | Menu 3 | Menu 4 | Function Description |
| --- | --- | --- | --- | --- |
| 1.DMX | Address | 001-512 |  | DMX Address Setting |
|  | Channels | 21Ch |  | DMX Channel Mode Selection |
|  |  | 24Ch |  |  |
| 2.Control | 1.Pan | 000-255 |  | X axis |
|  | 2.Pan Fine | 000-255 |  | X axis fine |
|  | 3.Tilt | 000-255 |  | Y axis |
|  | 4.Tilt Fine | 000-255 |  | Y axis fine |
|  | 5.P/T Speed | 000-255 |  | XY axis speed |
|  | 6.Dimmer | 000-255 |  | Dimming |
|  | 7.Strobe | 000-255 |  | Strobe |
|  | 8.Color | 000-255 |  | Color wheel |
|  | 9..Cyan | 000-255 |  | Cyan (C of CMY) |
|  | 10.Magenta | 000-255 |  | Magenta (M of CMY) |
|  | 11.Yellow | 000-255 |  | Yellow (Y of CMY) |
|  | 12.FixGobo | 000-255 |  | FixGobo |
|  | 13.Gobo | 000-255 |  | Gobo |
|  | 14.Gobo R | 000-255 |  | GoboR |
|  | 15.Iris | 000-255 |  | Iris |
|  | 16.Frost | 000-255 |  | Frost |
|  | 17.Prism | 000-255 |  | Prism |
|  | 18.Prism R | 000-255 |  | Prism-R |
|  | 19..LPrism | 000-255 |  | LPrism |
|  | 20.LPrism R | 000-255 |  | .LPrism R |
|  | 21.Focus | 000-255 |  | Focus |
|  | 22.Focus Fine | 000-255 |  | Focus Fine |
|  | 23.Zoom | 000-255 |  | Zoom |
| 3.Auto | 1-2 |  |  | Auto mode |
| 4.Slave | Yes |  |  | Slave Mode |
|  | No |  |  |  |
| 5.Network Config | 1.Local IP | 2.xxx.xxx.xxx |  | IP address setting |
|  | 2.Subnet Mask | 255.0.0.0 |  | IP subnet mask setting |
|  | 3.ArtNet Enable | On |  | ArtNet function setting |
|  |  | Off |  |  |
|  | 4.ArtNet Univ | Net | 000-127 | ARTNET universe setting (universe =Net*256+Sub*16+Port) |
|  |  | Sub | 000-015 |  |
|  |  | Port | 000-015 |  |
|  | 5.sACN Enable | On |  | sACN function setting |
|  |  | Off |  |  |
|  | 6.sACN Universe | 00001-63999 |  | sACN universe |
| 6.Settings | 1.Pan Angle | 540 |  | X axis 540 degree |
|  |  | 360 |  | X axis 360 degree |
|  | 2.Pan Dir | Normal |  | Pan Direction normal |
|  |  | Inverted |  | Pan Direction inverted |
|  | 3.Tilt Dir | Normal |  | Tilt Direction normal |
|  |  | Inverted |  | Tilt Direction inverted |
|  | 4.P/T Feedback | On |  | P/T feedback on |
|  |  | Off |  | P/T feedback off |
|  | 5.Dimmer Curve | 1.Linear |  | Dimming curves |
|  |  | 2.Square |  |  |
|  |  | 3.Inverse Square |  |  |
|  |  | 4.S-Curve |  |  |
|  | 6.Dimmer Speed | Fast |  | Fast dimming speed |
|  |  | Smooth |  | Smooth dimming speed |
|  | 7.Display Dir | Auto |  | Automatic screen display |
|  |  | Normal |  | Normal screen display |
|  |  | Inverted |  | Inverted screen display |
|  | 8.PWM Frequency | 1KHz |  | PWM frequency setting |
|  |  | 3KHz |  |  |
|  |  | 6KHz |  |  |
|  |  | 12KHz |  |  |
|  | 9.DMX Fail | 1.Off |  | NO DMX, fixture off |
|  |  | 2.Hold |  | NO DMX, signal holding |
|  |  | 3.Control |  | NO DMX, enter into control mode |
|  |  | 4.Auto |  | NO DMX, enter into auto mode |
|  | 10.Fan Mode | 1.Auto |  | Auto fan speed |
|  |  | 2.High |  | High fan speed |
|  |  | 3.Theatre |  | Theatre mode |
|  |  | 4.Low |  | Low fan mode |
|  |  | 5.Off |  | Fan off |
|  | 11.Backlight Time | 5s |  | Backlight time setting |
|  |  | 10s |  |  |
|  |  | 20s |  |  |
|  |  | 30s |  |  |
|  |  | Always On |  |  |
|  | 12.Key Backlight | On |  | Key backlight on |
|  |  | Off |  | Key backlight off |
|  | 13.Key Lock | Locked |  | Key locked |
|  |  | Unlocked |  | Key unlocked |
|  | 14.Calibrations (Password:0088) | 1.Pan | 000-255(127) | X axis calibration |
|  |  | 2.Tilt | 000-255(127) | Y axis calibration |
|  |  | 3.Color | 000-255(127) | Color wheel calibration settings |
|  |  | 4.Color W/1 | 000-255(127) | Color wheel W and 1 calibration |
|  |  | 5.Color 1/2 | 000-255(127) | Color wheel 1 and 2 calibration |
|  |  | 6.Color 2/3 | 000-255(127) | Color wheel 2 and 3 calibration |
|  |  | 7.Color 3/4 | 000-255(127) | Color wheel 3 and 4 calibration |
|  |  | 8.Color 4/5 | 000-255(127) | Color wheel 4 and 5 calibration |
|  |  | 9.Color 5/6 | 000-255(127) | Color wheel 5 and 6 calibration |
|  |  | 10.Cyan | 000-255(127) | Cyan (C of CMY) |
|  |  | 11.Magenta | 000-255(127) | Magenta (M of CMY) |
|  |  | 12.Yellow | 000-255(127) | Yellow (Y of CMY) |
|  |  | 13.FixGobo | 000-255(127) | Fixed gobo wheel calibration |
|  |  | 14.Gobo | 000-255(127) | Gobo wheel calibration |
|  |  | 15.Gobo R | 000-255(127) | Gobo rotation calibration |
|  |  | 16.Gobo1 R | 000-255(127) | Gobo 1 rotation calibration |
|  |  | 17.Gobo2 R | 000-255(127) | Gobo 2 rotation calibration |
|  |  | 18.Gobo3 R | 000-255(127) | Gobo 3 rotation calibration |
|  |  | 19.Gobo4 R | 000-255(127) | Gobo 4 rotation calibration |
|  |  | 20.Gobo5 R | 000-255(127) | Gobo 5 rotation calibration |
|  |  | 21.Gobo6 R | 000-255(127) | Gobo 6 rotation calibration |
|  |  | 22.Gobo7 R | 000-255(127) | Gobo 7 rotation calibration |
|  |  | 23.Gobo8 R | 000-255(127) | Gobo 8 rotation calibration |
|  |  | 24.Gobo9 R | 000-255(127) | Gobo 9 rotation calibration |
|  |  | 25.Iris | 000-255(127) | Iris calibration |
|  |  | 26.Frost | 000-255(127) | Frost calibration |
|  |  | 27.Prism | 000-255(127) | Prism calibration |
|  |  | 28.Prism R | 000-255(127) | Prism rotation calibration |
|  |  | 29.LPrism | 000-255(127) | Linear prism calibration |
|  |  | 30.LPrism R | 000-255(127) | Linear prism rotation calibration |
|  |  | 31.Focus | 000-255(127) | Focus calibration |
|  |  | 32.Zoom | 000-255(127) | Zoom calibration |
|  |  | 33.FocusFixG1 | 000-255(127) | Focus fixed gobo 1 calibration |
|  |  | 34.FocusFixG2 | 000-255(127) | Focus fixed gobo 2 calibration |
|  |  | 35.FocusFixG3 | 000-255(127) | Focus fixed gobo 3 calibration |
|  |  | 36.FocusFixG4 | 000-255(127) | Focus fixed gobo 4 calibration |
|  |  | 37.FocusFixG5 | 000-255(127) | Focus fixed gobo 5 calibration |
|  |  | 38.FocusFixG6 | 000-255(127) | Focus fixed gobo 6 calibration |
|  |  | 39.FocusFixG7 | 000-255(127) | Focus fixed gobo 7 calibration |
|  |  | 40.FocusFixG8 | 000-255(127) | Focus fixed gobo 8 calibration |
|  |  | 41.FocusFixG9 | 000-255(127) | Focus fixed gobo 9 calibration |
|  |  | 42.FocusFixG10 | 000-255(127) | Focus fixed gobo 10 calibration |
|  |  | 43.FocusFixG11 | 000-255(127) | Focus fixed gobo 11 calibration |
|  |  | 44.FocusFixG12 | 000-255(127) | Focus fixed gobo 12 calibration |
|  |  | 45.FocusFixG13 | 000-255(127) | Focus fixed gobo 1 3 calibration |
|  |  | 46.FocusGobo1 | 000-255(127) | Focus gobo 1 calibration |
|  |  | 47.FocusGobo2 | 000-255(127) | Focus gobo 2 calibration |
|  |  | 48.FocusGobo3 | 000-255(127) | Focus gobo 3 calibration |
|  |  | 49.FocusGobo4 | 000-255(127) | Focus gobo 4 calibration |
|  |  | 50.FocusGobo5 | 000-255(127) | Focus gobo 5 calibration |
|  |  | 51.FocusGobo6 | 000-255(127) | Focus gobo 6 calibration |
|  |  | 52.FocusGobo7 | 000-255(127) | Focus gobo 7 calibration |
|  |  | 53.FocusGobo8 | 000-255(127) | Focus gobo 8 calibration |
|  |  | 54.FocusGobo9 | 000-255(127) | Focus gobo 9 calibration |
|  |  | 55.Default | Yes/No | Restore default |
|  | 15.Reset Motor | 1.All Reset | Yes | All motor reset |
|  |  |  | No |  |
|  |  | 2.Pan Tilt Reset | Yes | Pan Tilt motor reset |
|  |  |  | No |  |
|  |  | 3.Focus Reset | Yes | Focus reset |
|  |  |  | No |  |
|  |  | 4.Gobo Reset | Yes | Gobo Reset |
|  |  |  | No |  |
|  |  | 5.CMY Reset | Yes | CMY reset |
|  |  |  | No |  |
|  | 16.Factory Reset | Yes |  | Factory Reset |
|  |  | No |  |  |
| 7.Information | 1.Version | MCU S1 x.x/xxxx |  | MCU S1 (display board) version / date |
|  |  | MCU S2 x.x/xxxx |  | MCU S2 (XY axis motor) version /date |
|  |  | MCU S3 x.x/xxxx |  | MCU S3 (LED driver) version /date |
|  |  | MCU S4 x.x/xxxx |  | MCU S4 (CMY) version /date |
|  |  | MCU S5 x.x/xxxx |  | MCU S5 (gobo subassembly) version /date |
|  |  | MCU S6 x.x/xxxx |  | MCU S6 (focus, zoom, prism, frost) version /date |
|  | 2.Temperature | Led xx℃ |  | LED source temperature |
|  |  | Power xx℃ |  | Power temperature |
|  | 3.Fan Speed | PowerFan xxxx rpm |  | Power fan speed |
|  |  | Led Fan1 xxxx rpm |  | LED fan1 speed |
|  |  | Led Fan2 xxxx rpm |  | LED fan2 speed |
|  |  | Led Fan3 xxxx rpm |  | LED fan3 speed |
|  |  | Led Fan4 xxxx rpm |  | LED fan4 speed |
|  |  | Gobo Fan xxxx rpm |  | Gobo fan speed |
|  |  | Head Fan1 xxxx rpm |  | Head fan1 speed |
|  |  | Head Fan2 xxxx rpm |  | Head fan2 speed |
|  | 4.Time | PowerTime xxxxh |  | Power time |
|  |  | Led Time xxxxh |  | LED timne |
|  | 5.RDM UID | xxxx: xxxxxxxx |  | RDM UID |
|  | 6.Network MAC | xx xx xx xx xx xx |  | Network MAC |
|  | 7.Error Status | xxxx |  | Error Status |

Note 1: After the display reversed, the up and down keys of the buttons are reversed (the up and down movements are in the same direction as the arrow), and the functions of the SET key and MODE key remain unchanged.

Note 2: Go to “Information”, and then press the SET key for 5 seconds to clear the light duration.

- DXM Channel

| Function | DMX Value | Setting | 21CH | 24CH |
| --- | --- | --- | --- | --- |
| Pan | 000-255 | 0-100% | 1 | 1 |
| Pan Fine | 000-255 | 0-100% |  | 2 |
| Tilt | 000-255 | 0-100% | 2 | 3 |
| Tilt Fine | 000-255 | 0-100% |  | 4 |
| P/T Speed | 000-255 | Fast to Slow | 3 | 5 |
| Dimmer | 000-255 | 0-100% | 4 | 6 |
| Strobe | 000-009 | Close | 5 | 7 |
|  | 010-127 | Linear Strobe slow to fast |  |  |
|  | 128-255 | Random Strobe slow to fast |  |  |
| Color | 000-007 | White | 6 | 8 |
|  | 008-015 | Color_1 |  |  |
|  | 016-023 | Color_2 |  |  |
|  | 024-031 | Color_3 |  |  |
|  | 032-039 | Color_4 |  |  |
|  | 040-047 | Color_5 |  |  |
|  | 048-055 | Color_6 |  |  |
|  | 056-063 | Color_7 |  |  |
|  | 064-071 | White + Color_1 |  |  |
|  | 072-079 | Color_1 + Color_2 |  |  |
|  | 080-087 | Color_2 + Color_3 |  |  |
|  | 088-095 | Color_3 + Color_4 |  |  |
|  | 096-103 | Color_4+ Color_5 |  |  |
|  | 104-111 | Color_5 + Color_6 |  |  |
|  | 112-127 | White |  |  |
|  | 128-191 | Roll fast to slow(from down to up) |  |  |
|  | 192-255 | Roll slow to fast(from up to down) |  |  |
| Cyan | 000-255 | 0-100% | 7 | 9 |
| Magenta | 000-255 | 0-100% | 8 | 10 |
| Yellow | 000-255 | 0-100% | 9 | 11 |
| FixGobo | 000-003 | Open | 10 | 12 |
|  | 004-007 | FixGobo1 |  |  |
|  | 008-011 | FixGobo2 |  |  |
|  | 012-015 | FixGobo3 |  |  |
|  | 016-019 | FixGobo4 |  |  |
|  | 020-023 | FixGobo5 |  |  |
|  | 024-027 | FixGobo6 |  |  |
|  | 028-031 | FixGobo7 |  |  |
|  | 032-035 | FixGobo8 |  |  |
|  | 036-039 | FixGobo9 |  |  |
|  | 040-043 | FixGobo10 |  |  |
|  | 044-047 | FixGobo11 |  |  |
|  | 048-051 | FixGobo12 |  |  |
|  | 052-055 | FixGobo13 |  |  |
|  | 056-059 | FixGobo1 shake slow to fast |  |  |
|  | 060-063 | FixGobo2 shake slow to fast |  |  |
|  | 064-067 | FixGobo3 shake slow to fast |  |  |
|  | 068-071 | FixGobo4 shake slow to fast |  |  |
|  | 072-075 | FixGobo5 shake slow to fast |  |  |
|  | 076-079 | FixGobo6 shake slow to fast |  |  |
|  | 080-083 | FixGobo7 shake slow to fast |  |  |
|  | 084-087 | FixGobo8 shake slow to fast |  |  |
|  | 088-091 | FixGobo9 shake slow to fast |  |  |
|  | 092-095 | FixGobo10 shake slow to fast |  |  |
|  | 096-099 | FixGobo11 shake slow to fast |  |  |
|  | 100-103 | FixGobo12 shake slow to fast |  |  |
|  | 104-107 | FixGobo13 shake slow to fast |  |  |
|  | 108-127 | Open |  |  |
|  | 128-191 | Clockwise roll fast to slow |  |  |
|  | 192-255 | Anticlockwise roll slow to fast |  |  |
| Gobo | 000-005 | Open | 11 | 13 |
|  | 006-011 | Gobo1 |  |  |
|  | 012-017 | Gobo2 |  |  |
|  | 018-023 | Gobo3 |  |  |
|  | 024-029 | Gobo4 |  |  |
|  | 030-035 | Gobo5 |  |  |
|  | 036-041 | Gobo6 |  |  |
|  | 042-047 | Gobo7 |  |  |
|  | 048-053 | Gobo8 |  |  |
|  | 054-059 | Gobo9 |  |  |
|  | 060-065 | Gobo1 shake slow to fast |  |  |
|  | 066-071 | Gobo2 shake slow to fast |  |  |
|  | 072-077 | Gobo3 shake slow to fast |  |  |
|  | 078-083 | Gobo4 shake slow to fast |  |  |
|  | 084-089 | Gobo5 shake slow to fast |  |  |
|  | 090-095 | Gobo6 shake slow to fast |  |  |
|  | 096-101 | Gobo7 shake slow to fast |  |  |
|  | 102-107 | Gobo8 shake slow to fast |  |  |
|  | 108-113 | Gobo9 shake slow to fast |  |  |
|  | 114-127 | Open |  |  |
|  | 128-191 | Clockwise roll fast to slow |  |  |
|  | 192-255 | Anticlockwise roll slow to fast |  |  |
| Gobo R | 000-127 | 0-360° | 14 | 16 |
|  | 128-191 | Clockwise roll fast to slow |  |  |
|  | 192-255 | Anticlockwise roll slow to fast |  |  |
| Iris | 000-210 | 0100% | 13 | 15 |
|  | 211-225 | Pulse Opening from Slow to fast |  |  |
|  | 226-240 | Pulse Closing from Slow to fast |  |  |
|  | 241-255 | Random Pulse form slow to fast |  |  |
| Frost | 000-255 | 0100% | 14 | 16 |
| Prism | 000-009 | Close | 15 | 17 |
|  | 010-255 | Open |  |  |
| Prism R | 000-127 | 0-360° | 16 | 18 |
|  | 128-191 | Clockwise roll fast to slow |  |  |
|  | 192-255 | Anticlockwise roll slow to fast |  |  |
| L Prism | 000-009 | Close | 17 | 19 |
|  | 010-255 | Open |  |  |
| L Prism R | 000-127 | 0-360° | 18 | 20 |
|  | 128-191 | Clockwise roll fast to slow |  |  |
|  | 192-255 | Anticlockwise roll slow to fast |  |  |
| Focus | 000-255 | 0100% | 19 | 21 |
| Focus Fine | 000-255 | 0100% |  | 22 |
| Zoom | 000-255 | 0100% | 20 | 23 |
| Function | 000-009 | No Function | 21 | 24 |
|  | 010-014 | Pan Angle 540 |  |  |
|  | 015-019 | Pan Angle 360 |  |  |
|  | 020-024 | Pan Normal |  |  |
|  | 025-029 | Pan Inverted |  |  |
|  | 030-034 | Tilt Normal |  |  |
|  | 035-039 | Tilt Inverted |  |  |
|  | 040-044 | P/T Feedback On |  |  |
|  | 045-049 | P/T Feedback Off |  |  |
|  | 050-054 | Dimmer Curve: Linear |  |  |
|  | 055-059 | Dimmer Curve: Square |  |  |
|  | 060-064 | Dimmer Curve: Inverse Square |  |  |
|  | 065-069 | Dimmer Curve: S-Curve |  |  |
|  | 070-074 | Dimmer Speed: Fast |  |  |
|  | 075-079 | Dimmer Speed: Slow |  |  |
|  | 080-104 | Reserved |  |  |
|  | 105-109 | Display Direction Auto |  |  |
|  | 110-114 | Display Direction Normal |  |  |
|  | 115-119 | Display Direction Inverted |  |  |
|  | 120-124 | PWM Frequency 1KHz |  |  |
|  | 125-129 | PWM Frequency 3KHz |  |  |
|  | 130-134 | PWM Frequency 6KHz |  |  |
|  | 135-139 | PWM Frequency 12KHz |  |  |
|  | 140-144 | Reserved |  |  |
|  | 145-149 | Fan Mode: Auto |  |  |
|  | 150-154 | Fan Mode: High |  |  |
|  | 155-159 | Fan Mode: Theatre |  |  |
|  | 160-164 | Fan Mode: Low |  |  |
|  | 165-169 | Fan Mode: Off |  |  |
|  | 170-199 | Reserved |  |  |
|  | 200-204 | All Reset |  |  |
|  | 205-209 | Pan Tilt Reset |  |  |
|  | 210-214 | Focus Reset |  |  |
|  | 215-219 | Gobo Reset |  |  |
|  | 220-224 | CMY Reset |  |  |
|  | 225-255 | Reserved |  |  |

- Operation and work mode description

1、Working mode switch

Press MODE or UP or DOWN to switch menu

1.DMX

2.Control

3.Auto

4.Slave

5.Network Config

6.Settings

7.Information

2、Working mode description

A、DMX mode

This is DMX mode, press SETUP to enter, press UP or DOWN to select, as follow:

| DMX |  |
| --- | --- |
| Address | 001~512 |
| Channels | 21CH/24CH |

B、Control mode

This is control mode, press SETUP to enter, press UP or DOWN to select, as follow:

| Control | 1.Pan | 000-255 | X axis |
| --- | --- | --- | --- |
|  | 2.Pan Fine | 000-255 | X axis fine |
|  | 3.Tilt | 000-255 | Y axis |
|  | 4.Tilt Fine | 000-255 | Y axis fine |
|  | 5.P/T Speed | 000-255 | XY axis speed |
|  | 6.Dimmer | 000-255 | Dimming |
|  | 7.Strobe | 000-255 | Strobe |
|  | 8.Color | 000-255 | Color1 |
|  | 9.Cyan | 000-255 | Cyan |
|  | 10.Magenta | 000-255 | Magenta |
|  | 11.Yellow | 000-255 | Yellow |
|  | 12.FixGobo | 000-255 | Fixed gobo |
|  | 13.Gobo | 000-255 | Rotating gobo |
|  | 14.Gobo R | 000-255 | Gobo rotation |
|  | 15.Iris | 000-255 | Iris |
|  | 16.Frost | 000-255 | Frost |
|  | 17.Prism | 000-255 | Prism |
|  | 18.Prism R | 000-255 | Prism rotation |
|  | 19.LPrism | 000-255 | Linear prism |
|  | 20.LPrism R | 000-255 | Linear prism rotation |
|  | 21.Focus | 000-255 | Focus |
|  | 22.Focus Fine | 000-255 | Focus Fine |
|  | 23.Zoom | 000-255 | Zoom |

C、Auto mode

This is auto mode, press SETUP to enter, press 1 or 2 to select built-in programs.

| Auto | 1 |
| --- | --- |
|  | 2 |

D、Slave mode

This is slave mode, press SETUP to enter, select YES, connect DMX cable, the fixture will work the same program with the main fixture.

| Slave | Yes |
| --- | --- |
|  | No |

E、Network Config

This is Network Config mode, press SETUP to enter, press UP or DOWN to select, as follow:

| Network Config | 1.Local IP | 2.xxx.xxx.xxx |  | IP address setting |
| --- | --- | --- | --- | --- |
|  | 2.Subnet Mask | 255.0.0.0 |  | IP subnet mask setting |
|  | 3.ArtNet Enable | On |  | ArtNet function setting |
|  |  | Off |  |  |
|  | 4.ArtNet Univ | Net | 000-127 | ARTNET universe setting（universe =Net*256+Sub*16+Port） |
|  |  | Sub | 000-015 |  |
|  |  | Port | 000-015 |  |
|  | 5.sACN Enable | On |  | sACN function setting |
|  |  | Off |  |  |
|  | 6.sACN Universe | 00001-63999 |  | sACN universe |

- IP address setting :press SETUP enter，press UP or DOWN to select, as follow:

| 1.Local IP | xxx,xxx,xxx,xxx |
| --- | --- |

- IP subnet mask setting: press SETUP to enter, press UP or DOWN to select, as follow:

| 2.Subnet Mask | 255,0,0,0 |
| --- | --- |

- ArtNet function setting: press SETUP to enter, press UP or DOWN to select, as follow:

| 3.ArtNet Enable | On |
| --- | --- |
|  | Off |

- ARTNET universe setting: press SETUP to enter, press UP or DOWN to select, as follow:

| 4.ArtNet Univ | Net | 000-127 |
| --- | --- | --- |
|  | Sub | 000-015 |
|  | Port | 000-015 |

ARTNET universe computing method: Net*256+Sub*16+Port

- sACN function setting: press SETUP to enter, press UP or DOWN to select, as follow:

| 5.sACN Enable | On |
| --- | --- |
|  | Off |

- sACN universe setting: press SETUP to enter, press UP or DOWN to select, as follow:

| 6.sACN Universe | 00001-63999 |
| --- | --- |

F、Settings

This is setting mode, press SETUP to enter, press UP or DOWN to select, as follow:

| Settings | 1. Pan Angle |
| --- | --- |
|  | 2. Pan Dir |
|  | 3. Tilt Dir |
|  | 4. P/T Feedback |
|  | 5. Dimmer Curve |
|  | 6. Dimmer Speed |
|  | 7. Display Dir |
|  | 8. PWM Frequency |
|  | 9. DMX Fail |
|  | 10. Fan Mode |
|  | 11. Backlight Time |
|  | 12. Key Backlight |
|  | 13. Key Lock |
|  | 14. Calibrations |
|  | 15. Motor Reset |
|  | 16. Factory Reset |

- X axis angle setting: press SETUP to enter, press UP or DOWN to select, as follow:

| Pan Angle | 540 |
| --- | --- |
|  | 360 |

- X axis inverted setting: press SETUP to enter, press UP or DOWN to select, as follow:

| Pan Dir | Normal | Pan direction normal |
| --- | --- | --- |
|  | Inverted | Pan direction inverted |

- Y axis inverted setting: press SETUP to enter, press UP or DOWN to select, as follow:

| Tilt Dir | Normal | Tilt direction normal |
| --- | --- | --- |
|  | Inverted | Tilt direction inverted |

- XY axis error correction setting: press SETUP to enter, press UP or DOWN to select, as follow:

XY axis

| P/T Feedback | On | Error correction on |
| --- | --- | --- |
|  | Off | Error correction off |

- There are 4 dimming curves, press SETUP to enter, press UP or DOWN to select, as follow:

| Dimmer Curve | 1. Linear |
| --- | --- |
|  | 2. Square |
|  | 3. Inverse Square |
|  | 4. S-Curve |

Curve 1：Linear                    Curve 2：Square

Curve 3：Inverse Square               Curve 4：S-Curve

- Dimmer Speed: press SETUP to enter, press UP or DOWN to select, as follow:

| Dimmer Speed | Fast | Dimmer speed fast |
| --- | --- | --- |
|  | Smooth | Dimmer speed smooth |

- Display direction setting: press SETUP to enter, press UP or DOWN to select, as follow:

| Display Dir | Auto | Display direction auto |
| --- | --- | --- |
|  | Normal | Display direction normal |
|  | Inverted | Display direction inverted |

- PWM frequency setting: press SETUP to enter, press UP or DOWN to select, as follow:

| PWM Frequency | 1KHz |
| --- | --- |
|  | 3KHz |
|  | 6KHz |
|  | 12KHz |

- DMX Fail: Console DMX signal clear, Press SETUP to enter, and press UP or DOWN to select, as follow:

| DMX Fail | 1. Off | (When DMX signal is failed, the fixture is off) |
| --- | --- | --- |
|  | 2. Hold | (When DMX signal is failed, the fixture will keep current state) |
|  | 3. Control | (When DMX signal is failed, it will change to control mode) |
|  | 4. Auto | (When DMX signal is failed, it will change to auto mode) |

- Fan mode setting: press SETUP to enter, press UP or DOWN to select, as follow:

| Fan Mode | 1. Auto | Auto fan speed |
| --- | --- | --- |
|  | 2. High | High fan speed |
|  | 3.Theatre | Theatre mode |
|  | 4. Low | Low fan mode |
|  | 5. Off | Fan off |

- Backlight Time setting: press SETUP to enter, press UP or DOWN to select, as follow:

| Backlight Time | 5s |
| --- | --- |
|  | 10s |
|  | 20s |
|  | 30s |
|  | Always On |

- Key Backlight setting: press SETUP to enter, press UP or DOWN to select, as follow:

| Key Backlight | On |
| --- | --- |
|  | Off |

- Key lock setting: press SETUP to enter, press UP or DOWN to select, as follow:

| Key Lock | Locked |
| --- | --- |
|  | Unlocked |

- Calibration setting: press SETUP to enter, press UP or DOWN to select, as follow:

| Calibrations (Password:0088) | 1.Pan | 000-255(127) | X axis calibration |
| --- | --- | --- | --- |
|  | 2.Tilt | 000-255(127) | Y axis calibration |
|  | 3.Color | 000-255(127) | Color wheel calibration |
|  | 4.Color W/1 | 000-255(127) | Color wheel W and 1 calibration |
|  | 5.Color 1/2 | 000-255(127) | Color wheel 1 and 2 calibration |
|  | 6.Color 2/3 | 000-255(127) | Color wheel 2 and 3 calibration |
|  | 7.Color 3/4 | 000-255(127) | Color wheel 3 and 4 calibration |
|  | 8.Color 4/5 | 000-255(127) | Color wheel 4 and 5 calibration |
|  | 9.Color 5/6 | 000-255(127) | Color wheel 5 and 6 calibration |
|  | 10.Cyan | 000-255(127) | Cyan (C of CMY) |
|  | 11.Magenta | 000-255(127) | Magenta (M of CMY) |
|  | 12.Yellow | 000-255(127) | Yellow (Y of CMY) |
|  | 13.FixGobo | 000-255(127) | Fixed gobo wheel calibration |
|  | 14.Gobo | 000-255(127) | Gobo wheel calibration |
|  | 15.Gobo R | 000-255(127) | Gobo rotation calibration |
|  | 16.Gobo1 R | 000-255(127) | Gobo 1 rotation calibration |
|  | 17.Gobo2 R | 000-255(127) | Gobo 2 rotation calibration |
|  | 18.Gobo3 R | 000-255(127) | Gobo 3 rotation calibration |
|  | 19.Gobo4 R | 000-255(127) | Gobo 4 rotation calibration |
|  | 20.Gobo5 R | 000-255(127) | Gobo 5 rotation calibration |
|  | 21.Gobo6 R | 000-255(127) | Gobo 6 rotation calibration |
|  | 22.Gobo7 R | 000-255(127) | Gobo 7 rotation calibration |
|  | 23.Gobo8 R | 000-255(127) | Gobo 8 rotation calibration |
|  | 24.Gobo9 R | 000-255(127) | Gobo 9 rotation calibration |
|  | 25.Iris | 000-255(127) | Iris calibration |
|  | 26.Frost | 000-255(127) | Frost calibration |
|  | 27.Prism | 000-255(127) | Prism calibration |
|  | 28.Prism R | 000-255(127) | Prism rotation calibration |
|  | 29.LPrism | 000-255(127) | Linear prism calibration |
|  | 30.LPrism R | 000-255(127) | Linear prism rotation calibration |
|  | 31.Focus | 000-255(127) | Focus calibration |
|  | 32.Zoom | 000-255(127) | Zoom calibration |
|  | 33.FocusFixG1 | 000-255(127) | Focus fixed gobo 1 calibration |
|  | 34.FocusFixG2 | 000-255(127) | Focus fixed gobo 2 calibration |
|  | 35.FocusFixG3 | 000-255(127) | Focus fixed gobo 3 calibration |
|  | 36.FocusFixG4 | 000-255(127) | Focus fixed gobo 4 calibration |
|  | 37.FocusFixG5 | 000-255(127) | Focus fixed gobo 5 calibration |
|  | 38.FocusFixG6 | 000-255(127) | Focus fixed gobo 6 calibration |
|  | 39.FocusFixG7 | 000-255(127) | Focus fixed gobo 7 calibration |
|  | 40.FocusFixG8 | 000-255(127) | Focus fixed gobo 8 calibration |
|  | 41.FocusFixG9 | 000-255(127) | Focus fixed gobo 9 calibration |
|  | 42.FocusFixG10 | 000-255(127) | Focus fixed gobo 10 calibration |
|  | 43.FocusFixG11 | 000-255(127) | Focus fixed gobo 11 calibration |
|  | 44.FocusFixG12 | 000-255(127) | Focus fixed gobo 12 calibration |
|  | 45.FocusFixG13 | 000-255(127) | Focus fixed gobo 1 3 calibration |
|  | 46.FocusGobo1 | 000-255(127) | Focus gobo 1 calibration |
|  | 47.FocusGobo2 | 000-255(127) | Focus gobo 2 calibration |
|  | 48.FocusGobo3 | 000-255(127) | Focus gobo 3 calibration |
|  | 49.FocusGobo4 | 000-255(127) | Focus gobo 4 calibration |
|  | 50.FocusGobo5 | 000-255(127) | Focus gobo 5 calibration |
|  | 51.FocusGobo6 | 000-255(127) | Focus gobo 6 calibration |
|  | 52.FocusGobo7 | 000-255(127) | Focus gobo 7 calibration |
|  | 53.FocusGobo8 | 000-255(127) | Focus gobo 8 calibration |
|  | 54.FocusGobo9 | 000-255(127) | Focus gobo 9 calibration |
|  | 55.Default | Yes/No | Restore default |

- Reset Motor setting: press SETUP to enter, press UP or DOWN to select, as follow:

| Reset Motor | 1.All Reset | Yes | All motor reset |
| --- | --- | --- | --- |
|  |  | No |  |
|  | 2.XY Reset | Yes | XY motor reset |
|  |  | No |  |
|  | 3.Focus Reset | Yes | Focus reset |
|  |  | No |  |
|  | 4.Gobo Reset | Yes | Gobo Reset |
|  |  | No |  |
|  | 5.CMY Reset | Yes | CMY reset |
|  |  | No |  |

- Factory restore setting: press SETUP to enter, press UP or DOWN to select, as follow:

This is for restore factory setting, after perform the function, all data restore initial state, except for calibration data and working time.

| Factory Reset | Yes |
| --- | --- |
|  | No |

F、Information

This is information mode to store some relevant information, as follow:

| Information | 1.Version | MCU S1 x.x/xxxx | MCU S1 (display board) version / date |
| --- | --- | --- | --- |
|  |  | MCU S2 x.x/xxxx | MCU S2 (XY axis motor) version /date |
|  |  | MCU S3 x.x/xxxx | MCU S3 (LED driver) version /date |
|  |  | MCU S4 x.x/xxxx | MCU S4 (CMY) version /date |
|  |  | MCU S5 x.x/xxxx | MCU S5 (gobo subassembly) version /date |
|  |  | MCU S6 x.x/xxxx | MCU S6 (focus, zoom, prism, frost) version /date |
|  | 2.Temperature | Led xx℃ | LED source temperature |
|  |  | Power xx℃ | Power temperature |
|  | 3.Fan Speed | PowerFan xxxx rpm | Power fan speed |
|  |  | Led Fan1 xxxx rpm | LED fan1 speed |
|  |  | Led Fan2 xxxx rpm | LED fan2 speed |
|  |  | Led Fan3 xxxx rpm | LED fan3 speed |
|  |  | Led Fan4 xxxx rpm | LED fan4 speed |
|  |  | Gobo Fan xxxx rpm | Gobo fan speed |
|  |  | Head Fan1 xxxx rpm | Head fan1 speed |
|  |  | Head Fan2 xxxx rpm | Head fan2 speed |
|  | 4.Time | PowerTime xxxxh | Power time |
|  |  | Led Time xxxxh | LED timne |
|  | 5.RDM UID | xxxx: xxxxxxxx | RDM UID |
|  | 6.Network MAC | xx xx xx xx xx xx | Network MAC |
|  | 7.Error Status | xxxx | Error Status |

Special instructions:

1. Enter information mode, when on time setting option, long press SETUP 5S and enter 0088 to clear all time.

2. Error Status: if the error come, when the display screen shows Error, after unlock, press Enter to enter the error status menu to check the information, as below:

| Error information | Function |
| --- | --- |
| Pan | X axis restore error |
| Tilt | Y axis restore error |
| Color | Color wheel restore error |
| Cyan | Cyan(C of CMY ) restore error |
| Magenta | Magenta(M of CMY ) restore error |
| Yellow | Yellow(Y of CMY ) restore error |
| FixGobo | Fixgobo restore error |
| Gobo | Gobo restore error |
| Gobo R | Gobo rotation restore error |
| Prism | Prism restore error |
| Prism R | Prism rotation restore error |
| LPrism | Linear prism restore error |
| LPrism R | Linear prism rotation restore error |
| Focus | Focus restore error |
| Zoom | Zoom restore error |
| LedTemp | LED temperature restore error |
| LedFan1 | LED fan1 error |
| LedFan2 | LED fan2 error |
| LedFan3 | LED fan3 error |
| LedFan4 | LED fan4 error |
| GoboFan | Gobo fan error |
| HeadFan1 | Head fan1 error |
| HeadFan2 | Head fan2 error |
| PowerFan | Power fan error |
| MCU S2 | MCU S2 error (XY axis motor board) |
| MCU S3 | MCU S3 error (Led driver board) |
| MCU S4 | MCU S4 error (CMY board) |
| MCU S5 | MCU S5 error (Gobo subassembly motor board) |
| MCU S6 | MCU S6 error (Focus motor board) |

- ArtNet:

| Receive | Respond | Function |
| --- | --- | --- |
| ArtPoll | ArtPollReply | Device Information |
| ArtDmx |  | DMX512 data output |
| ArtAddress | ArtPollReply | Revise universe and name |
| ArtIpProg | ArtIpProgReply | Revise IP and Subnet Mask |
| ArtTodRequest | ArtTodData | Check RDM UID list |
| ArtRdm | ArtRdm | RDM communication |

- RDM:

This fixture has RDM function. RDM is the protocol which support two-way traffic in standard DMX512. Configure and monitor fixture, like change DMX address, DMX mode and so on. Every fixture with RDM can be recognized by built-in UID code.

| Value | Parameter ID | Required | SET Allowed | Get Allowed |
| --- | --- | --- | --- | --- |
| 0x0001 | DISC_UNIQUE_BRANCH | * |  |  |
| 0x0002 | DISC_MUTE | * |  |  |
| 0x0003 | DISC_UN_MUTE | * |  |  |
| 0x0050 | SUPPORTED_PARAMETERS | * |  | * |
| 0x0051 | PARAMETER_DESCRIPTION | * |  | * |
| 0x0060 | DEVICE_INFO | * |  | * |
| 0x0080 | DEVICE_MODEL_DESCRIPTION |  |  | * |
| 0x0081 | MANUFACTURER_LABEL |  |  | * |
| 0x0082 | DEVICE_LABEL |  | * | * |
| 0x0090 | FACTORY_DEFAULTS |  | * |  |
| 0x00C0 | SOFTWARE_VERSION_LABEL | * |  | * |
| 0x00F0 | DMX_START_ADDRESS | * | * | * |
| 0x0200 | SENSOR_DEFINITION |  |  | * |
| 0x0201 | SENSOR_VALUE |  |  | * |
| 0x0202 | RECORD_SENSORS |  | * | * |
| 0x0500 | DISPLAY_INVERT |  | * | * |
| 0x1000 | IDENTIFY_DEVICE | * | * | * |

七、Notes

- If you have not done anything for 30s, it will lock screen automatically. Press “MODE” and “SETUP” for 3 seconds to unlock. In the unlocked state, press “MODE” and “SETUP” at the same time is to lock quickly.

- When display screen is inversed, press up arrow and down arrow to rollover.

- In order to avoid any problem, please follow the user manual to make sure it is in safety operation.

- When the fixture restore, if the motors are error, check them with the error information list.

- In order to avoid any problem, please follow the user manual to make sure it is in safety operation.

- Please don’t take apart the light when it is in charging.