XY-047-RGBA User Manual

Menu

| Menu 1 | Menu 2 | Menu 3 | Menu 3 | Functions |
| --- | --- | --- | --- | --- |
| 1.DMX | Address: | 001-512 |  | DMX Address |
|  | Channels: | 10/20-CH |  | DMX mode |
| 2. Manual | Pan: | 000-255 |  | Pan |
|  | Tilt:: | 000-255 |  | Tilt |
|  | Red1: | 000-255 |  | Red dimming |
|  | Green1: | 000-255 |  | Green dimming |
|  | Blue1: | 000-255 |  | Blue dimming |
|  | White1: | 000-255 |  | Warm white dimming |
|  | Red2: | 000-255 |  | Red dimming |
|  | Green2: | 000-255 |  | Green dimming |
|  | Blue2: | 000-255 |  | Blue dimming |
|  | White2: | 000-255 |  | Warm white dimming |
| 3.Auto | Pan: | 000-255 |  | Pan |
|  | Tilt: | 000-255 |  | Tilt |
|  | Motor Auto | On |  | Pan, Tilt Auto run angle |
|  |  | Off |  |  |
| 4.Program | Pan: | 000-255 |  | Pan |
|  | Tilt: | 000-255 |  | Tilt |
|  | Mode: | 1-15 |  | Built-in program |
|  | Color:(Mode:1) | 01-33 |  | Static color |
|  | Speed:(Mode:2-15) | 0-255 |  | Speed |
|  | Strobe: | 0-10 |  | No function |
|  |  | 11-127 |  | Linear strobe speed from slow to fast |
|  |  | 128-255 |  | Random strobe speed from slow to fast |
|  | Motor Auto | On |  | Pan, Tilt Auto run angle |
|  |  | Off |  |  |
| 5.Sound | Pan: | 000-255 |  | Pan |
|  | Tilt: | 000-255 |  | Tilt |
|  | Mode: | 01-06 |  | Sound mode |
|  | Sensitivity | 0-255(50) |  | Sound control sensitivity |
|  | Motor Auto | On |  | Pan, Tilt Auto run angle |
|  |  | Off |  |  |
| 6.Slave | Yes/No |  |  | Slave mode |
| 7.Settings | 1.Pan Angle | 540/360 |  | Pan Angle |
|  | 2.Pan Inv | Normal/Inverted |  | Pan Inverted |
|  | 3.Tilt Inv | Normal/Inverted |  | Tilt Inverted |
|  | 4.Curves Select | Linear |  | Dimming curves |
|  |  | Square |  |  |
|  |  | Inverse Square |  |  |
|  |  | S-Curve |  |  |
|  | 5.Dimmer Speed | Fast/ Smooth |  | Dimmer Speed |
|  | 6.Display Inv | Normal/Inverted |  | Display direction |
|  | 7.PWM Frequency | 12KHz/6KHz/3KHz/1KHz |  | Dimming frequency |
|  | 8.Dmx Fail | Off/Hold/Manual/Program |  | DMX signal |
|  | 9.Fan Mode | Auto |  |  |
|  |  | High |  |  |
|  |  | Low |  |  |
|  |  | Off |  |  |
|  | 10.Backlight Time | 5s/10s/20s/30s/Always on |  | Backlight Time |
|  | 11.Key Lock | Locked/ Unlocked |  | Key Lock |
|  | 12.Calibration |  | Pan: 000-255(127) | Pan motor calibration |
|  |  |  | Tilt: 000-255(127) | Till motor calibration |
|  |  |  | Default:Yes/No | Restore Defaults |
|  | 13.Reset Motor | Yes/No |  | Motor reset |
|  | 14.Factory Reset | Yes/No |  | Factory Reset |
| 8.Information | 1.Version | Vx.x / 0627 |  | Version / Date |
|  | 2.Temperature | LED:xx℃ |  | Temperature display |
|  | 3.Time | PowerTime:xxxxxh |  | Power Time |
|  |  | Led Time:xxxxxh |  | Led Time |
|  | 4.RDM UID | UID:xxxxxxxxxxxx |  | Fixture UID |
|  | 5. Fan Inf | State:XXXX |  | Fan information |
|  |  | Speed:XXXXrpm |  |  |

DMX Channel

| Function | DMX Value | Setting |  | 10-Ch | 20-Ch |
| --- | --- | --- | --- | --- | --- |
| Pan | 000-255 | 0-540° |  | 1 | 1 |
| Pan Fine | 000-255 | 0~100% |  |  | 2 |
| Tilt | 000-255 | 0-270° |  | 2 | 3 |
| Tilt Fine | 000-255 | 0~100% |  |  | 4 |
| P/T Speed | 000-255 | Speed from fast to slow |  | 3 | 5 |
| Dimmer | 000-255 | 0~100% |  | 4 | 6 |
| Strobe | 0-10 | No function |  | 5 | 7 |
|  | 11-127 | Linear strobe speed from slow to fast |  |  |  |
|  | 128-255 | Random strobe speed from slow to fast |  |  |  |
| Red All | 000-255 | 0~100% |  | 6 |  |
| Green All | 000-255 | 0~100% |  | 7 |  |
| Blue All | 000-255 | 0~100% |  | 8 |  |
| Amber All | 000-255 | 0~100% |  | 9 |  |
| Red 1 | 000-255 | 0~100% |  |  | 12 |
| Green 1 | 000-255 | 0~100% |  |  | 13 |
| Blue 1 | 000-255 | 0~100% |  |  | 14 |
| Amber 1 | 000-255 | 0~100% |  |  | 15 |
| Red 2 | 000-255 | 0~100% |  |  | 16 |
| Green 2 | 000-255 | 0~100% |  |  | 17 |
| Blue 2 | 000-255 | 0~100% |  |  | 18 |
| Amber 2 | 000-255 | 0~100% |  |  | 19 |
| Color preset | 000-024 | No function |  |  | 8 |
|  | 025-031 | Color1(R255 G000 B000 W000) |  |  |  |
|  | 032-038 | Color2(R255 G000 B000 W100) |  |  |  |
|  | 039-045 | Color3(R255 G000 B000 W200) |  |  |  |
|  | 046-052 | Color4(R255 G050 B000 W000) |  |  |  |
|  | 053-059 | Color5(R255 G150 B000 W000) |  |  |  |
|  | 060-066 | Color6(R255 G255 B000 W000) |  |  |  |
|  | 067-073 | Color7(R255 G255 B000 W0750) |  |  |  |
|  | 074-080 | Color8(R000 G255 B000 W255) |  |  |  |
|  | 081-087 | Color9(R000 G255 B000 W150) |  |  |  |
|  | 088-094 | Color10(R000 G255 B000 W050) |  |  |  |
|  | 095-101 | Color11(R000 G255 B000 W000) |  |  |  |
|  | 102-108 | Color12(R000 G255 B050 W000) |  |  |  |
|  | 109-115 | Color13(R000 G255 B150 W000) |  |  |  |
|  | 116-122 | Color14(R000 G255 B255 W000) |  |  |  |
|  | 123-129 | Color15(R000 G255 B255 W075) |  |  |  |
|  | 130-136 | Color16(R000 G255 B255 W150) |  |  |  |
|  | 137-143 | Color17(R000 G100 B255 W255) |  |  |  |
|  | 144-150 | Color18(R000 G000 B255 W100) |  |  |  |
|  | 151-157 | Color19(R000 G000 B255 W050) |  |  |  |
|  | 158-164 | Color20(R000 G000 B255 W000) |  |  |  |
|  | 165-171 | Color21(R075 G000 B255 W000) |  |  |  |
|  | 172-178 | Color22(R160 G000 B255 W000) |  |  |  |
|  | 179-185 | Color23(R255 G000 B255 W000) |  |  |  |
|  | 186-192 | Color24(R255 G000 B175 W000) |  |  |  |
|  | 193-199 | Color25(R255 G000 B100 W000) |  |  |  |
|  | 200-206 | Color26(R255 G000 B100 W050) |  |  |  |
|  | 207-213 | Color27(R255 G000 B025 W050) |  |  |  |
|  | 214-220 | Color28(R255 G000 B025 W025) |  |  |  |
|  | 221-227 | Color29(R255 G000 B025 W000) |  |  |  |
|  | 228-234 | Color30(R000 G000 B000 W255) |  |  |  |
|  | 235-241 | Color31(R075 G075 B000 W255) |  |  |  |
|  | 242-248 | Color32(R000 G000 B100 W255) |  |  |  |
|  | 249-255 | Color33(R255 G255 B255 W255) |  |  |  |
| Color running | 000-017 | No function |  |  | 9 |
|  | 018-034 | Jump1 |  |  |  |
|  | 035-051 | Jump2 |  |  |  |
|  | 052-068 | Jump3 |  |  |  |
|  | 069-085 | Jump4 |  |  |  |
|  | 086-102 | Jump5 |  |  |  |
|  | 103-119 | Jump6 |  |  |  |
|  | 120-136 | Jump7 |  |  |  |
|  | 137-153 | Fade1 |  |  |  |
|  | 154-170 | Fade2 |  |  |  |
|  | 171-187 | Fade3 |  |  |  |
|  | 188-204 | Fade4 |  |  |  |
|  | 205-221 | Fade5 |  |  |  |
|  | 222-238 | Fade6 |  |  |  |
|  | 239-255 | Fade7 |  |  |  |
| speed | 0-255 | Speed from slow to fast | Color running |  | 10 |
|  |  | Sound mode sensitivity 0-255 | Sound Mode |  |  |
| Sound | 000-032 | No function |  |  | 11 |
|  | 033-255 | Sound Mode |  |  |  |
| Function (if dmx value is present for 5 seconds) | 000-015 | No function |  | 10 | 20 |
|  | 016-030 | Pan Normal |  |  |  |
|  | 031-045 | Pan Inverted |  |  |  |
|  | 046-060 | Tilt Normal |  |  |  |
|  | 061-075 | Tilt Inverted |  |  |  |
|  | 076-090 | Dimmer Fast |  |  |  |
|  | 091-105 | Dimmer Smooth |  |  |  |
|  | 106-120 | Linear |  |  |  |
|  | 121-135 | Square |  |  |  |
|  | 136-150 | Inverse Square |  |  |  |
|  | 151-165 | S-Curve |  |  |  |
|  | 166-180 | PWM Frequency 12KHz |  |  |  |
|  | 181-195 | PWM Frequency 6KHz |  |  |  |
|  | 196-210 | PWM Frequency 3KHz |  |  |  |
|  | 211-225 | PWM Frequency 1KHz |  |  |  |
|  | 226-240 | Reset Motor |  |  |  |
|  | 241-255 | No function |  |  |  |

Operation and work mode description

- Working mode switch

Press MODE or UP or DOWN to switch menu

1.DMX

2. Manual

3.Auto

4.Program

5.Sound

6.Slave

7.Settings

8.Information

- Working mode description

a、DMX mode

This is DMX mode, press SETUP to enter, press UP or DOWN to select, as follow:

| DMX |  |
| --- | --- |
| Address : | 001~512 |
| Channels: | 10/20-CH |

b、Manual mode

This is Dimmer mode, press SETUP to enter, press UP or DOWN to select, as follow:

| 2. Manual | Pan: | 000-255 |  | Pan |
| --- | --- | --- | --- | --- |
|  | Tilt:: | 000-255 |  | Tilt |
|  | Red1: | 000-255 |  | Red dimming |
|  | Green1: | 000-255 |  | Green dimming |
|  | Blue1: | 000-255 |  | Blue dimming |
|  | White1: | 000-255 |  | Warm white dimming |
|  | Red2: | 000-255 |  | Red dimming |
|  | Green2: | 000-255 |  | Green dimming |
|  | Blue2: | 000-255 |  | Blue dimming |
|  | White2: | 000-255 |  | Warm white dimming |

c、Auto run mode

This is Auto run mode, it runs in cycles according to the speed and strobe set by the built-in program Program Mode: 02~ Program Mode: 15

| Auto | Pan: | 000-255 |
| --- | --- | --- |
|  | Tilt: | 000-255 |
|  | Motor Auto | On |
|  |  | Off |

d、Built-in Program Mode

This is built-in program, 15 built-in programs in total, press SETUP to enter, press UP or DOWN to select, as follow:

- Mode:01 Static color program

| Mode:01 | Pan: | 000-255 |
| --- | --- | --- |
|  | Tilt: | 000-255 |
|  | Color : | 01~33 |
|  | Strobe: | 0~10 |
|  |  | 11-127 |
|  |  | 128-255 |
|  | Motor Auto | On |
|  |  | Off |

2、 Mode:02~15  Built-in program

| Mode:02~15 | Pan: | 000-255 |
| --- | --- | --- |
|  | Tilt: | 000-255 |
|  | Speed: | 001~255 |
|  | Strobe: | 0~10 |
|  |  | 11-127 |
|  |  | 128-255 |
|  | Motor Auto | On |
|  |  | Off |

e、Sound mode

This is sound mode program, press the SETUP to enter, press UP or DOWN to select, as follow:

| Sound | Pan: | 000-255 |
| --- | --- | --- |
|  | Tilt: | 000-255 |
|  | Mode: | 01-06 |
|  | Sensitivity | 0-255 |
|  | Motor Auto | On |
|  |  | Off |

f、Slave mode

This is Slave mode. After connecting the DMX cable, the fixture will run with the main fixture at the same step.

| Slave | Yes |
| --- | --- |
|  | No |

g、Settings mode

This is Setting mode, press SETUP to enter, press UP or DOWN to select, as follow:

| Settings |  | 1.Pan Angle |
| --- | --- | --- |
|  |  | 2.Pan Inv |
|  |  | 3.Tilt Inv |
|  |  | 4.Curves Select |
|  |  | 5.Dimmer Speed |
|  |  | 6.Display Inv |
|  |  | 7.PWM Frequency |
|  |  | 8.Dmx Fail |
|  |  | 9.Fan Mode |
|  |  | 10.Backlight Time |
|  |  | 11.Key Lock |
|  |  | 12.Calibration |
|  |  | 13.Reset Motor |
|  |  | 14. Factory Reset |

1：Pan Angle setting，press SETUP to enter，press UP/DOWN to select，as follow:

| Pan Angle | 540 |
| --- | --- |
|  | 360 |

2：Pan Inv setting，press SETUP to enter，press UP/DOWN to select，as follow:

| Pan Inv | Normal |
| --- | --- |
|  | Inverted |

3：Tilt Invsetting，press SETUP to enter，press UP/DOWN to select，as follow:

| Tilt Inv | Normal |
| --- | --- |
|  | Inverted |

4：Dimming curve setting

There are 4 dimming curves, press SETUP to enter, press UP or DOWN to select, as follow:

| Curves Select | Linear |
| --- | --- |
|  | Square |
|  | Inverse Square |
|  | S-Curve |

a、linear Dimming

b、Square Law Dimming

c、Inverse Square Law Dimming

d、S-curve Dimming

5：immer Speed, press SETUP to enter, press UP or DOWN to select, as follow:

| Dimmer Speed | Fast |
| --- | --- |
|  | Smooth |

6：Display direction setting

| Display | Normal |
| --- | --- |
|  | Inverted |

7：PWM Frequency setting

| PWM Frequency | 12KHz |
| --- | --- |
|  | 6KHz |
|  | 3KHz |
|  | 1KHz |

8：DMX Fail: Console DMX signal clear, Press SETUP to enter, and press UP or DOWN to select, as follow:

| DMX Fail | Off | (When DMX signal connection is failed, the fixture is off) |
| --- | --- | --- |
|  | Hold | (When DMX signal connection is failed, the fixture will keep current state) |
|  | Manual | (When DMX signal connection is failed, it will change to Dimmer mode) |
|  | Program | (When DMX signal connection is failed, it will change to Built-in program mode) |

9：Fan Mode setting

| Fan Mode | Auto |
| --- | --- |
|  | High |
|  | Low |
|  | Off |

10：Backlight Time setting

| Backlight Time | 5S |
| --- | --- |
|  | 10S |
|  | 20S |
|  | 30S |
|  | Always on |

11：Key Lock

| Key Lock | Locked |
| --- | --- |
|  | Unlocked |

12：Calibration

| Calibration | Pan | 000-255 |
| --- | --- | --- |
|  | Tilt | 000-255 |
|  | Default | Yes/No |

13：Reset Motor

| Reset Motor | Yes |
| --- | --- |
|  | No |

14：Factory

This is for restore factory setting, after perform the function, all data restore initial state, press SETUP to enter, and press UP or DOWN to select, as follow：

| Factory | Yes |
| --- | --- |
|  | No |

h、Information mode

This is information mode to store some relevant information, as follow:

| Information | 1.Version | Vx.x / 0627 |
| --- | --- | --- |
|  | 2.Temperature | LED:xx℃ |
|  | 3.Time | PowerTime:xxxxxh |
|  |  | Led Time:xxxxxh |
|  | 4.RDM UID | UID:xxxxxxxxxxxx |
|  | 5. Fan Inf | State:XXXX |
|  |  | Speed:XXXXrpm |

Under this mode, long press SETUP 5S and enter 0088 to clear Work Time

IR Remote Control Instructions

Tips: When using the remote control, please aim at the receiving part, the operation is more flexible

- ON/OFF

This is the ON/OFF function key

- AUTO

This is Auto run mode

- PRO

Press this button to enter the built-in program mode, press "+" or "-" to select the built-in program "Program 01-22"

- SOUND

No function

- SPEED

In the built-in program mode Program 02-22, press this button to enter the speed setting. Press the "+" or "-"button to set the program running speed, "Speed  01-100"

- STROBE

In the built-in program mode, press this button to enter the strobe. If it is in the strobe state and then press this button, the strobe will stop, otherwise the strobe will start. Press "+" or "-" key to set strobe value, "Strobe 00-99"

- DIMMING

Press this button to enter the dimming mode, press one of the button "R", "G", "B", "W", and then press "+" or "-" "can be set for monochrome brightness value "0~255"

- 0~9

The numeric button represents a static color in non-DIMMING mode. When not in the setting state, press the number to corresponding to static color.

| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UV | purple | purple pink | orange | white | red | green | blue | yellow | Warm white |

Press “R”“G”“B”“A”“W” corresponding to Red, green, blue, amber, white

* Remote control to restore factory settings

First press the "OFF" button and then continuously press the "9", "8" and "7" buttons to restore the factory settings.

RDM(Remote Device Management)

The product support RDM protocol, RDM is a protocol that supports two-way traffic in standard DMX512 protocols，monitor light run by RDM control Equipment, configuration, and status, such as change light DMX address or mode and so on. Each light compatible RDM recognition by unique UID code.

| Value | Parameter ID | Required | SET Allowed | Get Allowed |
| --- | --- | --- | --- | --- |
| 0x0001 | DISC_UNIQUE_BRANCH | * |  |  |
| 0x0002 | DISC_MUTE | * |  |  |
| 0x0003 | DISC_UN_MUTE | * |  |  |
| 0x0050 | SUPPORTED_PARAMETERS | * |  | * |
| 0x0051 | PARAMETER_DESCRIPTION | * |  | * |
| 0x0060 | DEVICE_INFO | * |  | * |
| 0x0080 | DEVICE_MODEL_DESCRIPTION |  |  | * |
| 0x0081 | MANUFACTURER_LABEL |  |  | * |
| 0x0082 | DEVICE_LABEL |  | * | * |
| 0x0090 | FACTORY_DEFAULTS |  | * |  |
| 0x00C0 | SOFTWARE_VERSION_LABEL | * |  | * |
| 0x00F0 | DMX_START_ADDRESS | * | * | * |
| 0x0200 | SENSOR_DEFINITION |  |  | * |
| 0x0201 | SENSOR_VALUE |  |  | * |
| 0x0202 | RECORD_SENSORS |  | * | * |
| 0x0500 | DISPLAY_INVERT |  | * | * |
| 0x1000 | IDENTIFY_DEVICE | * | * | * |

Notice:

1.If you have not done anything for 30s, it will lock screen automatically. Press “MODE” and “SETUP” for 3 seconds unlock. In the unlocked state, press “MODE” and “SETUP” at the same time is to lock quickly (the lock of setting is on).

2. When products are power on, pressing the “MODE” and “SETUP” buttons at the same time. Then they can restore factory settings and the data will be initialized.

3. In order to avoid any problem, please follow the user manual to make sure it is in safety operation.

4. If the environment temperature is not in the using range, please reduce the using time of the full power to prevent products damaging because of overheating.

5. When the product is powered on, if the display Temperature indicates ×, or when the lock screen indicates ×℃, the temperature control is abnormal, please check whether the internal temperature control wire of the product is well connected to ensure the product can work normally.

6. Please don’t take apart the light when it is in charging.