KZ-005A User Manual V2.0

一、Introduction

- 5 menu buttons (face to the display screen, and the buttons are on its right side) : UP, DOWN, Left, Right, Middle button

| Up | Select the previous option |
| --- | --- |
| DOWN | Select the next option |
| Left button | Menu/Back to previous menu |
| Right button | Confirm the selection function |
| Middle button | Back to previous menu/Confirm the selection. Long press to unlock (when in lock status) |

二、Menu

| Menu 1 | Menu 2 | Menu 3 |  |  | Menu 4 | Menu 5 | Menu 6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1.Console | Channel | Save |  |  | Scene 1 …… Scene 20 |  |  |
|  |  | Load |  |  |  |  |  |
|  | Program | Program Scene |  |  | Scene 1 : 1-20 | Select Scene: NULL | Scene 1 …… Scene 20 |
|  |  |  |  |  | …… | Transition Time: 1000ms(0-1638375) |  |
|  |  |  |  |  | Scene 20 : 1-20 | Hode Time: 1000ms(0-1638375) |  |
|  |  | Run |  |  | Circle |  |  |
|  |  |  |  |  | Single |  |  |
|  |  |  |  |  | Stop |  |  |
|  |  |  |  |  | Run |  |  |
|  |  |  |  |  | Scene: 1-20 |  |  |
|  | Macro | Setting |  |  | Start Address | 001-512 |  |
|  |  |  |  |  | End Address | 001-512 |  |
|  |  |  |  |  | Interval | 001-100 |  |
|  |  |  |  |  | Mode | Mode1 |  |
|  |  |  |  |  |  | Mode2 |  |
|  |  | Static |  |  | Zero |  |  |
|  |  |  |  |  | Full |  |  |
|  |  |  |  |  | RGBW_R |  |  |
|  |  |  |  |  | RGBW_G |  |  |
|  |  |  |  |  | RGBW_B |  |  |
|  |  |  |  |  | RGBW_W |  |  |
|  |  |  |  |  | RGB_R |  |  |
|  |  |  |  |  | RGB_G |  |  |
|  |  |  |  |  | RGB_B |  |  |
|  |  |  |  |  | RGBALC_R |  |  |
|  |  |  |  |  | RGBALC_G |  |  |
|  |  |  |  |  | RGBALC_B |  |  |
|  |  |  |  |  | RGBALC_A |  |  |
|  |  |  |  |  | RGBALC_L |  |  |
|  |  |  |  |  | RGBALC_C |  |  |
|  |  |  |  |  | 2Slots_1 |  |  |
|  |  |  |  |  | 2Slots_2 |  |  |
|  |  | One By One |  |  | RGBW-Circle |  |  |
|  |  |  |  |  | RGBW-All |  |  |
|  |  |  |  |  | RGBW-R |  |  |
|  |  |  |  |  | RGBW-G |  |  |
|  |  |  |  |  | RGBW-B |  |  |
|  |  |  |  |  | RGBW-W |  |  |
|  |  |  |  |  | RGB_Circle |  |  |
|  |  |  |  |  | RGB_All |  |  |
|  |  |  |  |  | RGB-R |  |  |
|  |  |  |  |  | RGB-G |  |  |
|  |  |  |  |  | RGB-B |  |  |
|  |  |  |  |  | RGBALC_Circle |  |  |
|  |  |  |  |  | RGBALC_All |  |  |
|  |  |  |  |  | RGBALC_R |  |  |
|  |  |  |  |  | RGBALC_G |  |  |
|  |  |  |  |  | RGBALC_B |  |  |
|  |  |  |  |  | RGBALC_A |  |  |
|  |  |  |  |  | RGBALC_L |  |  |
|  |  |  |  |  | RGBALC_C |  |  |
|  |  |  |  |  | 2Slots_1 |  |  |
|  |  |  |  |  | 2Slots_2 |  |  |
|  |  |  |  |  | 1-Slot |  |  |
|  |  | Rainbow_RGB |  |  | Speed:001-010 |  |  |
|  |  | Rainbow_RGBW |  |  | Speed:001-010 |  |  |
|  |  |  |  |  | White:000-255 |  |  |
|  |  | Wheel |  |  | 1SLOT | 001-512 |  |
|  |  |  |  |  | …… |  |  |
|  |  |  |  |  | 6SLOT |  |  |
| 2.DMX IN |  |  |  |  |  |  |  |
| 3.RDM | xx)xx.xx.xx.xx.xx.xx | Slots: xx Start: xx V xx.xx.xx.xx |  |  |  |  |  |
|  |  | Start Address |  |  | 001-512 |  |  |
|  |  | Personality |  |  | 4-Slots |  |  |
|  |  |  |  |  | 6-Slots |  |  |
|  |  |  |  |  | 9-Slots |  |  |
|  |  | PID Get/Set |  |  | DMX_PERSONALITY | Details are shown in Table 3.1 below |  |
|  |  |  |  |  | DMX_PERSONALITY _ DESCRIPTION |  |  |
|  |  |  |  |  | RESET_DEVICE |  |  |
|  |  |  |  |  | SENSOR_VALUE |  |  |
|  |  |  |  |  | SENSOR_DEFINITION |  |  |
|  |  |  |  |  | SOFTWARE_VERSION _LABEL |  |  |
|  |  | Sensor |  |  | Sensor xx : xx | Type: xx Unit: xx Prefix: xx Range Min:xx Max:xx Normal Min:xx Max:xx Present Value:xx Lowest: xx Highest: xx |  |
| 4.Write Code | Address | 001-512 |  |  |  |  |  |
|  | Setting | AddrMode |  |  | 000-003 |  |  |
|  |  | AutoMode |  |  | 000-002 |  |  |
|  |  | Curvex |  |  | 000-003 |  |  |
|  |  | DimmerSpeed |  |  | 000-255 |  |  |
|  |  | Red Value |  |  | 000-255 |  |  |
|  |  | Green Value |  |  | 000-255 |  |  |
|  |  | Blue Value |  |  | 000-255 |  |  |
|  |  | White Value |  |  | 000-255 |  |  |
|  |  | Send Break |  |  | Off |  |  |
|  |  |  |  |  | ON |  |  |
|  | Write | Succeed/ Fault Count:xx |  |  |  |  |  |
|  | Read | Fault Count:0 |  |  |  |  |  |
|  |  | Device xx |  |  | Addr:xx AddrMode:xx AutoMode:xx Curvex:xx Dimmer:xx Temp:xx Voltage:xx Current:xx Watt:xx UID:xx.xx.xx.xx.xx.xx |  |  |
|  | UID | Last:xx.xx.xx.xx.xx.xx |  |  |  |  |  |
|  |  | Num:xxx |  |  |  |  |  |
|  |  | xx.xx.xx.xx.xx.xx |  |  |  |  |  |
|  |  | Set |  |  |  |  |  |
|  | Write Same | Succeed/ Fault Count:xx |  |  |  |  |  |
| 5.UCS512C4 Code | WriteAddr | DMX Address : xxx |  |  | 001-512 |  |  |
|  |  | Slot Count : xx |  |  | 001-004 |  |  |
|  |  | IC Count : xx |  |  | 001-032 |  |  |
|  |  | Write |  |  |  |  |  |
|  |  | Write Same |  |  |  |  |  |
|  | WriteParam | Byte Count : xx |  |  | 4 |  |  |
|  |  |  |  |  | 2 |  |  |
|  |  |  |  |  | 1 |  |  |
|  |  | R Value : xxx |  |  | 000-255 |  |  |
|  |  | G Value : xxx |  |  | 000-255 |  |  |
|  |  | B Value : xxx |  |  | 000-255 |  |  |
|  |  | W Value : xxx |  |  | 000-255 |  |  |
|  |  | Write |  |  |  |  |  |
| 6.Recorded | Record Program | File Name: xxxx File Size: xxxxMB Time:xx:xx:xx |  |  |  |  |  |
|  |  | Start | Stop |  |  |  |  |
|  | Play Program | Play Single |  |  | 1.BIN …… xx.BIN | File Name:xxxx File Size: xxxxMB Progress:xx% |  |
|  |  |  |  |  |  | Single |  |
|  |  |  |  |  |  | Loop |  |
|  |  |  |  |  |  | Stop |  |
|  |  | Play ALL |  |  | File Name:xxxx File Size:xxxxMB File:xx/xx |  |  |
|  |  |  |  |  | Single |  |  |
|  |  |  |  |  | Loop |  |  |
|  |  |  |  |  | Stop |  |  |
| 7.File | No |  |  |  |  |  |  |
| 8.ArtNet | Device | xx.xx.xx.xx |  |  |  |  |  |
| 9.Upgrade | xxxx.DS | DSxxxxxx Vxx.xx PCB:xx MCU:xx xx Data len = xxxxxx Upgrade? |  |  |  |  |  |
|  |  | Yes |  | No |  |  |  |
| 10.USB Storage |  |  |  |  |  |  |  |
| 11.Setting | Back Light | Disable |  |  |  |  |  |
|  |  | 01-60 |  |  |  |  |  |
|  | Artnet Enable | Disable |  |  |  |  |  |
|  |  | Enable |  |  |  |  |  |
|  | IP Mask | xxx.xxx.xxx.xxx |  |  |  |  |  |
|  | Local IP | xxx.xxx.xxx.xxx |  |  |  |  |  |
|  | Target IP | xxx.xxx.xxx.xxx |  |  |  |  |  |
|  | Universe | 00000-32767 |  |  |  |  |  |
|  | Data View | 000-255 |  |  |  |  |  |
|  |  | 000-100% |  |  |  |  |  |
|  | CRMX | xxxx（状态提示） Power On |  |  |  |  |  |
|  |  | Status |  |  | xxxx（状态提示） Unlink |  |  |
|  |  |  |  |  | Link |  |  |
|  |  |  |  |  | Power Off |  |  |
|  |  | Protocol |  |  | xxxx（状态提示） W-DMX G3 |  |  |
|  |  |  |  |  | W-DMX G4S |  |  |
|  |  |  |  |  | CRMX |  |  |
|  |  | Mode |  |  | xxxx（状态提示） Unlink |  |  |
|  | Language | English |  |  |  |  |  |
|  |  | Русский |  |  |  |  |  |
|  | Default | No |  |  |  |  |  |
|  |  | Yes |  |  |  |  |  |
| 12.Information |  |  |  |  |  |  |  |

3. RDM：

| Menu 4 | Menu 5 |  |
| --- | --- | --- |
| DMX_PERSONALITY | GET PDL=0 | Response:(PDL=xx) Count = xx, Current = xx |
|  | SET PDL=1 | Response:(PDL=xx) |
|  | xx |  |
| DMX_PERSONALITY_ DESCRIPTION | GET PDL=1 | Response:(PDL=xx) Slots Count = xx DMX Mode xx – Slot xx |
|  | xx |  |
| RESET_DEVICE | SET PDL=1 | Response:(PDL=xx) |
|  | xx |  |
| SENSOR_VALUE | GET PDL=1 | Response:(PDL=xx) |
|  | xx |  |
| SENSOR_DEFINITION | GET PDL=1 | Response:(PDL=xx) |
|  | xx |  |
| SOFTWARE_VERSION_LABEL | GET PDL=0 | Response:(PDL=xx) |

三、Product operation and function mode

3.1、Switching function mode

Press the up, down, left and right button or slide the screen up and down to switch menu icons, then press the middle button or click on the screen menu icon to switch the menu interface. The operation as follows:

| 1.DMX Console | 2.DMX View | 3.RDM |
| --- | --- | --- |
| 4.Write Code | 5.UCS512C4 Code | 6. Recorded |
| 7. File System | 8. ArtNet | 9. Upgrade |
| 10. USB Storage | 11.Setting | 12.Information |

3.2、Functional mode description

1. DMX Console（Simple console）

This function is a simple console with 512 channel control, supports simple programming and operation, and has built-in macro commands, which can be used to test landscape lighting fixtures.

The operation as follows:

| Console | Channel |
| --- | --- |
|  | Program |
|  | Macro |

- Channel scene can be saved or loaded. A scene is a set of 512 channel values. The operation as follows:

| Channel | Save | Scene 1 …… Scene 20 | Save scene |
| --- | --- | --- | --- |
|  | Load |  | Load scene |

- Program supports programming and running scene programs, the operation as follows:

| Program | Program Scene | Scene 1:1-20 | Select Scene: Scene xx(1-20) | Scene 1 …… Scene 20 | Select scene |
| --- | --- | --- | --- | --- | --- |
|  |  | …… | Transition Time: 1000ms(0-1638375) |  | Set the transition time of the scene |
|  |  | Scene 20:1-20 | Hode Time: 1000ms(0-1638375) |  | Set the scene duration |
|  | Run | Circle |  |  | Loop |
|  |  | Single |  |  | single play |
|  |  | Stop |  |  | Stop play |
|  |  | Run |  |  |  |
|  |  | Scene: 1-20 |  |  |  |

- Macro is built-in macro command is used to test landscape lighting fixtures. The operation as follows:

| Macro |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Setting | Static | One By One | Rainbow_RGB | Rainbow_RGBW | Wheel |

a、Setting can set the start address and end address of the test, the running time interval and the light-off mode of the dynamic test. The operation as follows:

| Setting | Start Address | 001-512 | Start Address |
| --- | --- | --- | --- |
|  | End Address | 001-512 | End Address |
|  | Interval | 001-100 | Interval |
|  | Mode | Mode1 | lights up and goes out |
|  |  | Mode2 | After running it once, the running water light effect will turn off again. |

b、Static is a built-in static scene that can be used for monochrome testing of 4-color RGBW or 3-color RGB, full light and full off. The operation as follows:

| Static | Zero | Full | RGBW_R | RGBW_G | RGBW_B | RGBW_W | RGB_R | RGB_G | RGB_B |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | RGBALC_R | RGBALC_G | RGBALC_B | RGBALC_A | RGBALC_L | RGBALC_C | 2Slots_1 | 2Slots_2 |  |

c、One By One is built-in dynamic testing, single-color testing can be performed one by one. For four-color, three-color, and two-color single-color lamps, there is a circulating water effect. Combined with the Mode setting, it will turn off after it lights up or run through the water lamp effect and then turn off. The operation as follows :

| One By One | RGBW-Circle | RGBW -All | RGBW _R | RGBW _G | RGBW _B | RGBW _W | RGB_ Circle | RGB _All | RGB-R | RGB-G | RGB-B |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | RGBALC _Circle | RGBAL C_All | RGBA LC_R | RGBA LC_G | RGBA LC_B | RGBA LC_A | RGBA LC_L | RGBA LC_C | 2Slots_1 | 2Slots_2 | RGBA LC_L |

d、Rainbow_RGB is a three-color lamp bead, and the special effect switching speed can be set. The operation as follows:

| Rainbow_RGB | Speed:001-010 |
| --- | --- |

e、Rainbow_RGBW is a four-color lamp bead. You can set the special effect switching speed and the brightness of the white light.

The operation as follows:

| Rainbow_RGBW | Speed:001-010 |
| --- | --- |
|  | White:000-255 |

f、Wheel can set the channel value at a certain distance to 255.

The operation as follows:

| Wheel | 1SLOT | 001-512 |
| --- | --- | --- |
|  | 2SLOT |  |
|  | 3SLOT |  |
|  | 4SLOT |  |
|  | 5SLOT |  |
|  | 6SLOT |  |

- DMX View（DMX512 Data monitoring）

This function is connected to the DMX line of the on-site lighting network through a DMX line. It can detect the DMX512 signal sent by the console to control the lighting and display the 512 channel values in real time. Additionally, ArtDMX signals can be detected.

3. RDM（RDM Device management）

This function is an RDM device management function. It searches for devices that support RDM through the DMX line. The operation as follows:

| RDM | xx) xx.xx.xx.xx.xx.xx | Slots: xx Start: xx V xx.xx.xx.xx |
| --- | --- | --- |
|  |  | Start Address |
|  |  | Personality |
|  |  | PID Get/Set |
|  |  | Sensor |

1)  Start Address is the starting address of RDM communication. The operation as follows:

| Start Address | 001-512 | Address value |
| --- | --- | --- |

- Personality is the DMX channel value of the lamp.

The operation as follows:

| Personality |  |  |
| --- | --- | --- |
| 4-Slots | 6-Slots | 9-Slots |

3)  PID Get/Set

| PID Get/Set | DMX_PERSONALITY | DMX channel |
| --- | --- | --- |
|  | DMX_PERSONALITY_ DESCRIPTION | DMX channel description |
|  | RESET_DEVICE | RESET |
|  | SENSOR_VALUE | SENSOR Value |
|  | SENSOR_DEFINITION | SENSOR description |
|  | SOFTWARE_VERSION_LABEL | SOFTWARE version label |

a、DMX_PERSONALITY can set up and obtain the DMX channel. The operation as follows:

| DMX_PERSONALITY | GET PDL=0 | Response:(PDL=xx) Count = xx, Current = xx |
| --- | --- | --- |
|  | SET PDL=1 | Response:(PDL=xx) |
|  | xx |  |

b、DMX_PERSONALITY_ DESCRIPTION can obtain the channel,The operation as follows:

| DMX_PERSONALITY_ DESCRIPTION | GET PDL=1 | Response:(PDL=xx) Slots Count = xx DMX Mode xx – Slot xx |
| --- | --- | --- |
|  | xx |  |

c、RESET_DEVICE can reset the fixture in RDM communication. The operation as follows:

| RESET_DEVICE | SET PDL=1 | Response:(PDL=xx) |
| --- | --- | --- |
|  | xx |  |

d、SENSOR_VALUE can obtain the sensor value. The operation as follows:

| SENSOR_VALUE | GET PDL=1 | Response:(PDL=xx) |
| --- | --- | --- |
|  | xx |  |

e、SENSOR_DEFINITION can get the description of the sensor. The operation as follows:

| SENSOR_DEFINITION | GET PDL=1 | Response:(PDL=xx) |
| --- | --- | --- |
|  | xx |  |

f、SOFTWARE_VERSION_LABEL can obtain the lighting software version in RDM communication. The operation as follows:

| SOFTWARE_VERSION_LABEL | GET PDL=0 | Response:(PDL=xx) |
| --- | --- | --- |

4)  Sensor is the sensor data of the communication device

| Sensor | Sensor xx : xx | Type: xx Unit: xx Prefix: xx Range Min:xx Max:xx Normal Min:xx Max:xx Present Value:xx Lowest: xx Highest: xx | Show the sensor’s type/unit/prefix/range size |
| --- | --- | --- | --- |

4. Write Code（Landscape lighting coding）

This function is a code writing function for landscape lamps. It is used to read and write parameters of our company's landscape products. The operation as follows:

| Write Code | Address |
| --- | --- |
|  | Setting |
|  | Write |
|  | Read |
|  | UID |
|  | Write Same |

- Address set the coding address, the operation as follows:

| Address | 001-512 |
| --- | --- |

- Setting is write code settings, the operation as follows:

| Setting | AddrMode | 0.RGBW*1 | Addr. Mode |
| --- | --- | --- | --- |
|  |  | 1.BSRGBW |  |
|  |  | 2.BSRGBWF |  |
|  |  | 3.RGBW*N |  |
|  | AutoMode | 0.Auto | Auto Mode |
|  |  | 1.ON |  |
|  |  | 2.DMX512 |  |
|  | Curvex | 0.linear | Curvex |
|  |  | 1.Square Law |  |
|  |  | 2.Inv Square Law |  |
|  |  | 3.S-Type |  |
|  | DimmerSpeed | 000-255 | Dimmer Speed |
|  | Red Value | 000-255 | Red Value |
|  | Green Value | 000-255 | Green Value |
|  | Blue Value | 000-255 | Blue Value |
|  | White Value | 000-255 | White Value |
|  | Send Break | Off | Disable sending code writing Break |
|  |  | ON | Can sending code writing Break |

- Writes parameters to the device. The operation as follows:

| Write | Succeed Count:xx | Writing parameters successfully Show number of devices |
| --- | --- | --- |
|  | Fault Count:xx | Failed to write parameters Show number of devices |

- Read parameters from the device, the operation is as follows:

| Read | Fault Count:0 |  | Read parameters successfully Show number of devices |
| --- | --- | --- | --- |
|  | Device xx | Addr : xx AddrMode : xx AutoMode : xx Curvex : xx Dimmer : xx Temp : xx Voltage : xx Current : xx Watt : xx UID : xx.xx.xx.xx.xx.xx | Show the reading device’s address code/address mode/Auto mode/dimmer/Voltage/current/watt/UID |

- UID sets the UID code used for RDM communication on the lamp. The operation as follows:：

| UID | Last : xx.xx.xx.xx.xx.xx | show device UID and quantity |
| --- | --- | --- |
|  | Num : xxx |  |
|  | xx.xx.xx.xx.xx.xx | Set up UID |
|  | Set |  |

- Write Same :after writing the code, the address code of each lamp will be the same. The operation is as follows:

| Write Same | Succeed Count:xx | Succeed Show number of devices |
| --- | --- | --- |
|  | Fault Count:xx | Fault Show number of devices |

5. UCS512C4 Code

This function is develop for LED driver UCS512C4 to write the code .The operation as follows:

| 5.UCS512C4 Code | WriteAddr | DMX Address : xxx | 001-512 | Code writing adress |
| --- | --- | --- | --- | --- |
|  |  | Slot Count : xx | 001-004 | Number of channels |
|  |  | IC Count : xx | 001-032 | IC quantity |
|  |  | Write |  | Write the code. Then the address code of each lamp will be different. |
|  |  | Write Same |  | Write the code. Then the address code of each lamp will be the same. |
|  | WriteParam | Byte Count : xx | 4 | Number of bytes |
|  |  |  | 2 |  |
|  |  |  | 1 |  |
|  |  | R Value : xxx | 000-255 | Red value |
|  |  | G Value : xxx | 000-255 | Green value |
|  |  | B Value : xxx | 000-255 | Blue value |
|  |  | W Value : xxx | 000-255 | White value |
|  |  | Write |  | Write the code |

6. Recorded（录播）

This function can recorded the light show effect.The operation as follows:

| Recorded | Record Program |
| --- | --- |
|  | Play Program |

Record Program. The operation as follows:

| Record Program | File Name: xxxx File Size: xxxxMB Time:xx:xx:xx |  | Display the file name, file size and time of the recording program. |
| --- | --- | --- | --- |
|  | Start | Stop | Select to start or stop the record program |

Play Program.The operation as follows:

| Play Program | Play Single | 1.BIN …… xx.BIN | File Name:xxxx File Size:xxxxMB Progress:xx% | Display the file name, file size and progress of the play single |
| --- | --- | --- | --- | --- |
|  |  |  | Single | Play single |
|  |  |  | Loop | Play in a loop |
|  |  |  | Stop | Stop to play |
|  | Play ALL | File Name:xxxx File Size:xxxxMB File:xx/xx |  | Display the file name, file size, file serial and file amount of the play program |
|  |  | Single |  | Play all file once |
|  |  | Loop |  | Play all file in a loop |
|  |  | Stop |  | Stop to play |

7. File

No

8. ArtNet

This controller connects ArtNet console through network cable, connects fixtures through DMX cable,and can output channel value from console to control fixtures.

| ArtNet | Device | xx.xx.xx.xx | Display the UID of the connected device |
| --- | --- | --- | --- |

9．Upgrade

Select the file and connect the device by DMX cable, enter to wait for the upgrade, and follow the direction to upgrade or reset the device. The operation as follows:

| Upgrade | xxxx.DS | DSxxxxxx Vxx.xx PCB:xx MCU:xx xx Data len = xxxxxx Upgrade? |  | Display the information of this program file |
| --- | --- | --- | --- | --- |
|  |  | Yes | No | Upgrade or not |
|  |  | Upgrade myself? |  | upgrading the device itself, a reminder will be displayed |

10. USB Storage（SD card reader）

Connect to the computer via USB, display the storage device on the computer, and you can copy the program files that need to be upgraded to the SD card.

11. Setting

The operation as follows:

| Setting | BackLight |
| --- | --- |
|  | Artnet Enable |
|  | IP Mask |
|  | Local IP |
|  | Target IP |
|  | Universe |
|  | Data View |
|  | CRMX |
|  | Language |
|  | Default |

- BackLight为背光时间设置，disable（显示屏常亮）,1-60（无操作1-60min后显示屏背光灭），操作如下：

| BackLight | Disable |
| --- | --- |
|  | 01-60 |

- Artnet Enable为Artnet网络开关设置，操作如下：

| Artnet Enable | Disable |
| --- | --- |
|  | Enable |

- IP Mask为子网掩码，操作如下：

| IP Mask | xxx.xxx.xxx.xxx |
| --- | --- |

- Local IP为本地IP，操作如下：

| Local IP | xxx.xxx.xxx.xxx |
| --- | --- |

- Target IP为目标IP，操作如下：

| Target IP | xxx.xxx.xxx.xxx |
| --- | --- |

- Universe为ArtNet协议的域，操作如下：

| Universe | 00000-32767 |
| --- | --- |

- Data View为输入数据查看格式（DMX IN数据），操作如下：

| Data View | 000-255 |
| --- | --- |
|  | 000-100% |

- CRMX为CRMX通信设置，操作如下：

| CRMX | xxxx（状态提示） Power On | 显示当前CRMX模块的工作状态，打开CRMX的电源 |
| --- | --- | --- |
|  | Status | 控制与显示CRMX模块的工作状态 |
|  | Mode | 控制与显示CRMX模块的工作模式 |
|  | Protocol | 切换CRMX的通信协议 |

- Status为显示与控制CRMX模块的工作状态，操作如下：

| Status | xxxx（状态提示） | 显示当前CRMX模块的工作 |
| --- | --- | --- |
|  | Unlink | 断开设备的CRMX通信连接 |
|  | Link | 连接与设备的CRMX通信 |
|  | Power Off | 关闭CRMX的电源 |

- Mode为控制CRMX模块的工作模式，操作如下：

| Mode | xxxx（状态提示） Switch Mode | 显示当前CRMX模块的工作模式，切换CRMX的工作状态 |
| --- | --- | --- |

- Protocol为切换控制CRMX通信协议，操作如下：

| Protocol | xxxx（状态提示） W-DMX G3 | 显示当前CRMX模块的通信协议，切换通信为W-DMX G3协议 |
| --- | --- | --- |
|  | W-DMX G4S | 切换通信为W-DMX G4S协议 |
|  | CRMX | 切换通信为CRMX协议 |

- Language为切换界面语言（英文/俄文）设置，操作如下：

| Language | English |
| --- | --- |
|  | Русский |

- Default为恢复出厂设置，执行此功能后，所有数据将恢复初始状态，操作如下：

| Default | No |
| --- | --- |
|  | Yes |

12. Information（信息）

显示本设备的版本号以及各功能的简单说明。

- Controller basic instructions

1. Press the up, down, left and right keys or slide the screen up and down to select the desired menu. When a menu is selected, the corresponding menu icon will be backlit.

2.At this time, press the right button or click on the selected menu to enter the lower-level menu. If you need to return to the upper-level menu, press the left button or click on the title at the top of the screen. If you need to return to the main menu interface, press the middle key.

3.Press the up and down keys or click the left and right yellow arrows to change the data size of the value. If the data is on the console, you can also use the fader to change the channel data size.