MYLED-126 User Manual

- Menu

| Menu 1 | Menu 2 | Menu 3 | Function |
| --- | --- | --- | --- |
| 1:DMX | 1)Address: | 001-512 | DMX Mode |
|  | 2)Channels: | 4/6/9-CH |  |
| 2:Slave |  |  | Slave mode |
| 3:AUTO |  |  | Auto run mode |
| 4:Program | 1)Mode: | 1-14 | Built-in Program |
|  | 2)Color:(Mode:1) | 0-15 | Static Color |
|  | 2)Speed:(Mode:2-14) | 1-100 | Speed |
|  | 3)Strobe: | 0-99 | Strobe |
| 5:Dimmer | 1)Red: | 000~255 | Dimming mode |
|  | 2)Green: | 000~255 |  |
|  | 3)Blue: | 000~255 |  |
|  | 4)White: | 000~255 |  |
| 6:Settings | 1)Curves Select: | 1- 4 | Dimming curve selection |
|  | 2)Dimmer Speed: | Fast/ Smooth | Dimming speed selection |
|  | 3)Dmx Fail: | Off/Hold | DMX signal option |
|  | 4)Dmx Sync: | On/Off | DMX Synchronization setting |
|  | 5)Lock |  | Standby lock screen |
|  | 6)Factory: | Yes/No | Factory Reset |
| 7:Information | Versions: Vx.x.x |  | Version display |
|  | Temperature: xx℃ |  | Temperature detection |
|  | UID:xxxxxxxxxxxx |  | Fixture UID |

- DMX Channels

| Function | DMX Value | Setting |  | 4 Channels | 6 Channels | 9 Channels |
| --- | --- | --- | --- | --- | --- | --- |
| Dimmer | 000-255 | 0~100% |  |  | 1 | 1 |
| Red | 000-255 | 0~100% |  | 1 | 2 | 2 |
| Green | 000-255 | 0~100% |  | 2 | 3 | 3 |
| Blue | 000-255 | 0~100% |  | 3 | 4 | 4 |
| White | 000-255 | 0~100% |  | 4 | 5 | 5 |
| Color Preset | 0~15 | No function |  |  |  | 6 |
|  | 16~31 | R(255)G(0)B(0)W(0) |  |  |  |  |
|  | 32~47 | R(0)G(255)B(0)W(0) |  |  |  |  |
|  | 48~63 | R(0)G(0)B(255)W(0) |  |  |  |  |
|  | 64~79 | R(0)G(0)B(0)W(255) |  |  |  |  |
|  | 80~95 | R(255)G(150)B(0)W(0) |  |  |  |  |
|  | 96~111 | R(255)G(180)B(0)W(0) |  |  |  |  |
|  | 112~127 | R(255)G(255)B(0)W(0) |  |  |  |  |
|  | 128~143 | R(255)G(0)B(255)W(0) |  |  |  |  |
|  | 144~159 | R(255)G(0)B(140)W(0) |  |  |  |  |
|  | 160~175 | R(0)G(255)B(255)W(0) |  |  |  |  |
|  | 176~191 | R(255)G(0)B(0)W(210) |  |  |  |  |
|  | 192~207 | R(0)G(255)B(0)W(210) |  |  |  |  |
|  | 208~223 | R(0)G(0)B(255)W(210) |  |  |  |  |
|  | 224~239 | R(255)G(200)B(40)W(90) |  |  |  |  |
|  | 240~255 | R(255)G(255)B(255)W(255) |  |  |  |  |
| Program | 0 | No function |  |  |  | 7 |
|  | 1~19 | Program 02 |  |  |  |  |
|  | 20~39 | Program 03 |  |  |  |  |
|  | 40~59 | Program 04 |  |  |  |  |
|  | 60~79 | Program 05 |  |  |  |  |
|  | 80~99 | Program 06 |  |  |  |  |
|  | 100~119 | Program 07 |  |  |  |  |
|  | 120~139 | Program 08 |  |  |  |  |
|  | 140~159 | Program 09 |  |  |  |  |
|  | 160~179 | Program 10 |  |  |  |  |
|  | 180~199 | Program 11 |  |  |  |  |
|  | 200~219 | Program 12 |  |  |  |  |
|  | 220~239 | Program 13 |  |  |  |  |
|  | 240~255 | Program 14 |  |  |  |  |
| Speed | 0-255 | Slow to fast | Program 02-14 |  |  | 8 |
| Strobe | 0-10 | No function |  |  | 6 | 9 |
|  | 11-255 | Slow to fast |  |  |  |  |

- Operation and Mode illustration

- Operation Mode Transformation

Press the MODE, UP or DOWN button to transform menus.

1:DMX

2:Slave

3:AUTO

4:Program

5:Dimmer

6:Settings

7:Information

- Mode illustration

a、DMX mode

Press the setup button to enter and select by the up or down button. As follows:

| DMX |  |
| --- | --- |
| Address : | 001~512 |
| Channels: | 4/6/9-ch |

b、SLAVE mode

Press the setup button to enter, choose YES. After connecting the DMX cable, the light will run with the main fixture at the same step.

c、AUTO mode

Press “Setup” to enter, choose YES. Lights will circularly run in according with the speed and strobe of built-in Program Mode:02~ Program Mode:14

d、Built-in Program

There are 14 built-in programs. Please press the setup button to enter and up or down button to select. As follows:

- Mode:01 Static color effect program

| Mode:01 | Color : | 00~15 |
| --- | --- | --- |
|  | Strobe: | 00~99 |

| 0.Black | 1.Red | 2.Green | 3.Blue | 4.White | 5.Amber | 6.Orange | .7Yellow |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 8.Purple | 9.Pink | 10.Cyan | 11.Light Red | 12.Light Green | 13.Light Blue | 14.Warm White | 15.Cold White |

- Mode:02~14Buil in program

| Mode:02~14 | Speed: | 001~100 |
| --- | --- | --- |
|  | Strobe: | 00~99 |

e、Dimmer mode

Please press the setup button to enter and select by the up or down button. As follows

| Dimmer | Red: | 000~255 |
| --- | --- | --- |
|  | Green: | 000~255 |
|  | Blue: | 000~255 |
|  | White: | 000~255 |

f、Settings

Please press the setup button to enter and select by the up or down button. As follows

| Settings | Curves Select |
| --- | --- |
|  | Dimmer Speed |
|  | DMX Fail |
|  | Dmx Sync |
|  | Lock |
|  | Factory |

1：Dimming curve settings

There are four available dimming curves. Please press the setup button to enter and select by the up or down button. As follows:

| Curves Select | 1 )linear |
| --- | --- |
|  | 2)Square Law |
|  | 3)Inv Square Law |
|  | 4)S-Type |

a、linear dimming

b、Square Law dimming

c、Inv Square Law dimming

d、S-curve dimming

2：Dimmer Speed Please press the setup button to enter and select by the up or down button. As follows：

| Dimmer Speed | Fast |
| --- | --- |
|  | Smooth |

3：DMX Fail console DMX signal clear.

Please press the setup button to enter and select by the up or down button. As follows：

| DMX Fail | Off | (When DMX signal connection fail, light off) |
| --- | --- | --- |
|  | Hold | (When DMX signal connection fail，light keeps current state) |

4：Dmx Sync

In DMX mode, several fixtures can also operate at the same step, even though no DMX control for a period of time. Please press the setup button to enter and select by the up or down button. As follows：

| Dmx Sync | On |
| --- | --- |
|  | Off |

5：Lock

| Lock | Yes |
| --- | --- |
|  | No |

6：Factory

All data will restore initial state after factory sets. Please press the setup button to enter and select by the up or down button. As follows:

| Factory | Yes |
| --- | --- |
|  | No |

g、Information

Store relevant information, as follows:

| Information | Versions: x.x.x |
| --- | --- |
|  | Temperature: xx℃ |
|  | UID:xxxxxxxxxxxx |

四、 RDM(Remote Device Management)

This fixture has RDM function. RDM is the protocol which support two-way traffic in standard DMX512. Configure and monitor fixture, like change DMX address, DMX mode and so on. Every fixture with RDM can be recognized by built-in UID code.

| Parameter ID | Discovery command | SET command | Get command |
| --- | --- | --- | --- |
| DISC_UNIQUE_BRANCH | * |  |  |
| DISC_MUTE | * |  |  |
| DISC_UN_MUTE | * |  |  |
| DEVICE_INFO |  |  | * |
| SOFTWARE_VERSION_LABEL |  |  | * |
| DMX_START_ADDRESS |  | * | * |
| IDENTIFY_DEVICE |  | * | * |
| SUPPORTED_PARAMETERS |  |  | * |
| SENSOR_DEFINITION |  |  | * |
| SENSOR_VALUE |  |  | * |
| DMX_PERSONALITY |  | * | * |
| DMX_PERSONALITY_DESCRIPTION |  |  | * |
| RESET_DEVICE |  | * |  |
| FACTORY_DEFAULTS |  | * |  |

五、 Notice

- If no press any button in 30s, it will lock screen automatically and show current operating mode, WDMX mode, temperature status and battery(this battery is reference value, changes for on and off is very large). Of course, it will unlock again by pressing the mode and setup button for 3s at the same time. Press mode and setup button at the same time to quickly lock under unlock status (Lock need to be YES in Setting).

- When the product power's on and shows version information, pressing the “MODE” and “SETUP” buttons for some seconds at the same time. After this, it can restore factory settings and the data will be initialized.

- In order to avoid any problem, please follow the user manual to make sure it is in safety operations.

- If the environment temperature is not in the using range, please reduce the using time of the full power to prevent products damaged because of overheating.

- When the product power is on, the LED display screen shows temperature “×”. Or it shows ×℃ when locking screen. It means temperature anomalies. Please check whether lines of internal temperature control connect well.

- In order to prevent the accident, please don’t tear down the light under the condition of charging.